- 0 OBJECTIVE

  - To describe a procedure for the operation of the Biostat B Fermenter

- 0 SCOPE

2.1 This procedure applies to the operation of Biostat B Fermenter, located in the R&D lab at Oncosimis Biotech Pvt Ltd., Plot no 3, IDA Uppal, Genpact Rd, Habsiguda, Hyderabad

- REFERENCE

- NIL

- MATERIAL & SAFETY EQUIPMENT

Materials:

- Fermentation medium

- Silicone tubings

- Parchment Paper

- Rubber bands

- pH electrode

- D.O. electrode

- Antifoam probe

- Aluminium foil

Equipment:

- Name: Fermenter.

- Make: Biostat B

- Number

- RESPONSIBILITY

- Research Associate :

Shall operate the Biostat B Fermenter as per the procedure.

- Senior Research Associate:

Ensures that the Biostat B Fermenter is operated as per the procedure.

- R & D Head :

Ensures that the SOI is being followed as per written instructions.

- PROCEDURE

Pre startup

6.1 Ensure that the Fermenter vessel is clean

6.2 Calibrate the pH probe with appropriate standard buffers 4.01, 7.00, Immerse the probe in pH 7.00 standard buffer for calibration, the procedure is repeated with pH 4.01 buffer

6.3 Check that the D.O. electrode membrane is intact and that it contains the required quantity of D.O. electrolyte

Vessel Assembly

6.4 Place the vessel on the console platform

6.5 Lubricate the vessel gasket and head O-ring with silicone grease

6.6 Place the head plate on the flange of the vessel and lock it to the clamping ring with knurled screws. Do not overtighten

6.7 Install four-way ports in the head plate. Be sure to use a blind plug when a port is    not in use

6.8 Pour the fermentation medium into the vessel through the blind port. Close the		 port

6.9 Install pH and D.O. electrodes

6.10 Install antifoam probe into the head plate. Cover the probe head with aluminum foil

6.11 Connect the inlet air filter to the air- sparger tube and the exhaust air- filter to the exhaust condenser

6.12 Close the inlet air filter tubing with a pinchcock and cover the end with parchment paper. 	Secure with rubber bands

6.13 Loosely cover the exhaust filter end with aluminum foil

6.14 Slide the silicone tubing for feeds onto the feed ports on the head plate and cover the ends with parchment paper. Secure with rubber bands

6.15 Place the housing cover on the top of the bearing housing

6.16 Disconnect all probes and remove probe cables if any

6.17 Cover the ends of pH & D.O. electrodes with protective caps. Cover the caps with aluminum foil

6.18 Remove the sampler rubber bulb, close the sampler valve

6.19 Cover the sample port with a parchment paper. Secure with rubber bands

6.20 Connect silicone tubing to the harvest/transfer port and cover its end with parchment paper. Secure with rubber bands.  Close the tubing with a pinch cock

6.21 Cover the motorhead with the aluminum foil

Vessel Sterilization

6.22 Place the complete Fermenter vessel assembly inside the autoclave

6.23 Sterilize by autoclaving at 121oC, 15psi

6.24 After the sterilization cycle, release the pressure built up in the autoclave only when the temperature has dropped below 90oC

Seed culture preparation

6.24 Use a conical flask with suitable media of required volume

6.25 Inoculate the seed culture and let it grow to the optimum cell density.

Inoculation of the culture vessel

6.26 Transfer the seed into the Fermenter through the inoculation port under aseptic conditions.

6.27 Close the port.

Sampling Procedure

6.28 Check to be sure that the sample bottle is loose and not sealed against the sample gasket

6.29 Close the pinch cock valve on the sampler tube, if open

6.30 Open the pinch cock valve and obtain the desired volume of sample. Close pinch cock valve

6.31 Remove sample from the sampler bottle by pushing sterile air through 0.22u syringe filter and collected in sterile test tubes for in-process analysis

Harvest

6.32 At the end of the fermentation run, harvest the batch

Shut down procedure

6.32 Shut off all the utilities (air, chilled water, and oxygen)

6.33 Set all the Fermenter controls to ‘OFF’

6.34 Turn ‘OFF’ the power supply

Cleaning

6.35 At the end of the fermenter run, after the broth is harvested, clean & decontaminate the fermenter and prepare it for a new run

Precautions

6.36 Handle the fermenter glass vessel with care to prevent breakage

6.37 Take precautions to prevent any spillage of the culture into the environment

6.38 In case of any spillage, disinfect the area immediately with 70% IPA solution

- ABBREVIATIONS

NIL

- ANNEXURES

NIL

9.0 SUMMARY OF CHANGES

| Change control No | Reasons for revision |
| --- | --- |
| NA | NEW |